GOKARAJU RANGARAJU INSTITUTE OF ENGINEERING AND TECHNOLOGY

AUTONOMOUS

This is to certify that the project titled

“SMART PASS BOX CONTROLLER’’

is a bonafide work carried out by the following students in partial fulfilment of the requirements for Advanced Academic Center intern, submitted to the chair, AAC during the academic year 2019-20.

| NAME | ROLL NO. | BRANCH |
| --- | --- | --- |
| G.KRISHNA MITHRA | 18241A04L2 | ECE |
| N.NIKHIL | 18241A04M0 | ECE |
| P.SAI TEJA | 18241A04M2 | ECE |
| P.BHARGAV | 18241A04M4 | ECE |
| P.ROHIT SAI | 18241A04M6 | ECE |

Dr/Ms./Mr.

Dr B R K Reddy                      Dr Ramamurthy Suri

Project Supervisor                    Program Coordinator                  Associate Dean, AAC

We express our deep sense of gratitude to our respected Director, Gokaraju Rangaraju Institute of Engineering and Technology for the valuable guidance and for permitting us to carry out this project.

With immense pleasure, we record our deep sense of gratitude to our respected principal, for permitting us to carry out this project.

We are thankful to Associate Dean, Advance Academic Centre for providing us appropriate ecosystem required for the project to complete.

We are thankful to our project supervisor who spared valuable time for us and influence novel ideas to guide us. I am indebted to all the above without whom I would not have concluded the project.

CONTENTS

- ABSTRACT                                                                    6

- EMBEDDED SYSTEMS

Introduction                                                    7

Applications                                                   8

- ARDUINO MEGA

Introduction                                                    9

Technical Specifications                               10

Arduino MEGA Pins                                    10

Advantages of Arduino Board                      11

Different types of Arduino Boards               11

Features of Arduino Boards                         12

Applications                                                  12

- ULTRASONIC SENSOR

Introduction                                                   13

Working                                                        13

Uses of Ultrasonic Sensor                             14

- ULTRAVIOLET LIGHT

Introduction                                                   15

Discovery                                                      16

Applications                                                  17

- ELECTROMAGNET

Introduction                                                   18

History                                                           19

Uses of Electromagnet                                  20

- RELAY

Introduction                                                   21

History                                                           22

Working Principle                                         22

Construction                                                  23

- SMART PASS BOX CONTROLLER

Introduction                                                  26

Types of Pass Boxes                                    26

Uniqueness of the Innovation                      27

Construction                                                 28

Working                                                       29

Block Diagram                                             30

Concept and Objectives                               30

Project code                                                 31

Applications                                                 34

ABSTRACT

Sterilization process refers to the elimination/removal of all form of                            biological agents present in a specified region. Sterilization can be achieved                    through various means such as heating, irradiation, high pressure and                      filtration. It has been one of the prominent process in any production. In                   previous times it has been very difficult to achieve sterilization mainly in the pharmaceutical area and the other industrial area, though the demand for this process never descended. Due to the development of the technology pass box is one of               the solutions for the Sterilization. This process had extended its roots to all                     the areas of the human life for example, in the medical industries there is a need to transfer the material form one room to another room without any change in the atmospheric conditions. If there is any change in the material’s physical or            chemical properties it affects the product. In such cases the Pass Box will be               very helpful for the transfer of the materials. The pass box is used between                    the classified and the unclassified areas. It works on the principle of                      interlocking door mechanism with UV present inside the box. It accepts                          the material from one side and undergoes a process called “Sterilization” in                   the presence of UV light. After the completion of the process the pure                      material can be collected from the other side of the box.

CHAPTER 1

EMBEDDED SYSTEMS

INTRODUCTION:

An embedded system is a controller with a dedicated function within a larger              mechanical or electrical system, often with real-time computing constraints.                     It is embedded as part of a complete device often including hardware and         mechanical parts. Embedded systems control many devices in common use today. Ninety-eight percent of all microprocessors manufactured are used in embedded    systems.

Modern embedded systems are often based on microcontrollers . Since the             embedded system is dedicated to specific tasks, design engineers can optimize                  it to reduce the size and cost of the product and increase the reliability and                 performance. Some embedded systems are mass-produced, benefiting from              economies of scale.

Embedded systems range from portable devices such as digital watches and                MP3 players, to large stationary installations like traffic lights, programmable            logic controllers, and largely complex systems like hybrid vehicles, MRI, and           avionics. Complexity varies from low, with a single microcontroller chip, to very        high with multiple units, peripherals and networks mounted inside a large            chassis or enclosure.

Embedded systems have become very important today as they control many of              the common devices we use. An embedded system is some combination of              computer hardware and software, either fixed in capability or programmable,                that is specifically designed for a particular kind of application device. Industrial machines, automobiles, medical equipment, cameras, household appliances, airplanes, vending machines, and toys.

APPLICATIONS:

Embedded systems are commonly found in consumer, industrial, automotive,         medical, commercial and military applications. Telecommunications systems          employ numerous embedded systems from telephone switches for the network to         cell phones at the end user. Computer networking uses dedicated routers and          network bridges to route data.

Household appliances such as microwave ovens, washing machines include       embedded systems to provide flexibility, efficiency and features. Advanced HVAC systems use networked thermostats to more accurately and     efficiently control temperature that can change by time of day and season. Home automation uses wired- and wireless-networking that can be used to control lights, climate, security, audio/visual, surveillance, etc., all of which use embedded devices      for sensing and controlling.

Transportation systems from flight to automobiles increasingly use embedded systems. Automobiles, electric vehicles, and hybrid vehicles increasingly use embedded systems to maximize efficiency and reduce pollution. Medical equipment uses embedded systems for vital signs monitoring, electronic stethoscopes for amplifying sounds,         and various medical imaging  for non-invasive internal inspections. Embedded       systems within medical equipment are often powered by industrial computers.

Embedded systems are used in transportation, fire safety, safety and security,         medical applications and life critical systems, as these systems can be isolated             from hacking. In dealing with security, the embedded systems can be self-sufficient      and be able to deal with cut electrical and communication systems.

CHAPTER 2

ARDUINO MEGA

INTRODUCTION:

Arduino Mega 2560 is a Microcontroller board based on Atmega2560. It comes           with more memory space and I/O pins as compared to other boards available in             the market. There are 54 digital I/O pins and 16 analog pins incorporated on                   the board that make this device unique and stand out from others. Out of 54              digital I/O, 15 are used for PWM (pulse width modulation). A crystal oscillator of 16MHz frequency is added on the board. This board comes with USB cable port           that is used to connect and transfer code from computer to the board. DC                   power jack is coupled with the board that is used to power the board. ICSP header            is a remarkable addition to Arduino Mega which is used for programming the        Arduino and uploading the code from the computer. This board comes with two     voltage regulator i.e. 5V and 3.3V which provides the flexibility to regulate the        voltage as per requirements. There is no much difference between Arduino Uno and Arduino Mega except later comes with more memory space, bigger size and more         I/O pins. Arduino software called Arduino IDE is used to program the board             which is a common software used for all boards belonged to Arduino family.   Availability of Atmega16 on the board makes it different than Arduino Pro Mini which uses USB to serial converter to program the board. There is a reset button and                   4 hardware serial port called USART which produces a maximum speed for             setting up communication.

TECHNICAL SPECIFICATIONS:

Microcontroller: ATmega2560

Operating Voltage:  5V

Input Voltage (recommended): 7-12V

Input Voltage (limits): 6-20V

Digital I/O Pins: 54 (of which 14 provide PWM output)

Analog Input Pins: 16

DC Current per I/O Pin: 40 mA

DC Current for 3.3V Pin: 50 mA

Flash Memory: 256 KB of which 8 KB used by bootloader

SRAM: 8 KB

EEPROM: 4 KB

Clock Speed: 16 MHz

ARDUINO MEGA PINS:

VIN:  The input voltage to the Arduino board when it's using an external power        source. We can supply voltage through this pin, or, if supplying voltage via the        power jack, access it through this pin.

5V: The regulated power supply used to power the microcontroller and other   components on the board. This can come either from VIN via an on-board           regulator, or be supplied by USB or another regulated 5V supply.

3V3: A 3.3 volt supply generated by the on-board regulator. Maximum current draw is 50 mA.

GND: Ground pins.

LED 13: There is a built-in LED connected to digital pin 13. When the pin is HIGH value, the LED is on, when the pin is LOW, it's off.

AREF:  Reference voltage for the analog inputs. Used with analogReference().

Reset: Bring this line LOW to reset the microcontroller. Typically used to add a reset button to shields which block the one on the

ADVANTAGES OF ARDUINO BOARD:

Arduino also makes simpler the working process of microcontroller, but it gives          some advantages over other systems like:

They are inexpensive

They provide cross-platform

Simple, clear programming environment

Open source and extensible software

Open source and extensible hardware

DIFFERENT TYPES OF ARDUINO BOARDS:

The list of Arduino boards includes the following such as:

Arduino Uno (R3)

LilyPad Arduino

Red Board

Arduino Mega (R3)

Arduino Leonardo

FEATURES OF ARDUINO BOARD:

| Arduino Board | Processor | Memory | Digital I/O | Analogue I/O |
| --- | --- | --- | --- | --- |
| Arduino Uno | 16Mhz ATmega328 | 2KB SRAM, 32KB flash | 14 | 6 input, 0 output |
| Arduino Due | 84MHz AT91SAM3X8E | 96KB SRAM, 512KB flash | 54 | 12 input, 2 output |
| Arduino Mega | 16MHz ATmega2560 | 8KB SRAM, 256KB flash | 54 | 16 input, 0 output |
| Arduino Leonardo | 16MHz ATmega32u4 | 2.5KB SRAM, 32KB flash | 20 | 12 input, 0 output |

APPLICATIONS:

Robotics and Control system

Agricultural Areas

Home Automation

Traffic Lights Control System

Water Level Indication

Security and Defense System

Parking Lot Counter

Medical, Industrial Automation

CHAPTER 3

ULTRASONIC SENSOR

INTRODUCTION:

Ultrasonic sensing is one of the best ways to sense proximity and detect levels            with high reliability. An ultrasonic sensor is an instrument that measures                        the distance to an object using ultrasonic sound waves. An ultrasonic sensor                 uses a transducer to send and receive ultrasonic pulses that relay back               information about an object’s proximity. High-frequency sound waves                     reflect from boundaries to produce distinct echo patterns.

WORKING:

Ultrasonic sound vibrates at a frequency above the range of human hearing.     Transducers are the microphones used to receive and send the ultrasonic                  sound. Our ultrasonic sensors, like many others, use a single transducer to send                 a pulse and to receive the echo.  The sensor determines the distance to a                     target by measuring time lapses between the sending and receiving of the             ultrasonic pulse.

USES OF ULTRASONIC SENSOR:

Ultrasound is reliable in any lighting environment and can be used inside or            outside.  Ultrasonic sensors can handle collision avoidance for a robot, and                 being moved often, as long as it isn’t too fast. Ultrasonic sensors are so widely           used that they can be reliably implemented in grain bin sensing applications,              water level sensing, drone applications and sensing cars at your local                          drive-thru restaurant or bank. Ultrasonic rangefinders are commonly used as           devices to detect a collision.

Ultrasonic Sensors are best used in the non-contact detection of:

Presence

Level

Position

Distance

Ultrasonic sensors are Independent of:

Light

Smoke

Dust

Color

Material

CHAPTER 4

ULTRAVIOLET LIGHT

INTRODUCTION:

Ultraviolet (UV) designates a band of the electromagnetic spectrum with          wavelength from 10 nm to 400 nm, shorter than that of visible light but longer          than X-rays. UV radiation is present in sunlight, and contributes about 10% of                the total output of the Sun. It is also produced by electric arcs and specialized              lights, such as mercury-vapor lamps, tanning lamps, and black lights.

Suntan and sunburn are familiar effects of over-exposure of the skin to UV,               along with higher risk of skin cancer. Living things on dry land would be               severely damaged by ultraviolet radiation from the Sun if most of it were not           filtered out by the Earth's atmosphere. More energetic, shorter-wavelength          "extreme" UV below 121 nm ionizes air so strongly that it is absorbed before                    it reaches the ground.

Ultraviolet is also responsible for the formation of bone-strengthening                      vitamin D in most land vertebrates, including humans. The UV spectrum thus                has effects both beneficial and harmful to human health.

The lower wavelength limit of human vision is conventionally taken as                          400 nm, so ultraviolet rays are invisible to humans, although some people                      can perceive light at slightly shorter wavelengths than this. Insects, birds,                       and some mammals can see  near-UV light.

DISCOVERY:

"Ultraviolet" means "beyond violet”, violet being the color of the highest          frequencies of visible light. Ultraviolet has a higher frequency than violet light.             UV radiation was discovered in 1801 when the German physicist Johann               Wilhelm Ritter observed that invisible rays just beyond the violet end of                         the visible spectrum darkened silver chloride-soaked paper more quickly than            violet light itself. He called them "oxidizing rays" to emphasize chemical        reactivity and to distinguish them from "heat rays", discovered the previous               year at the other end of the visible spectrum. The simpler term "chemical rays"            was adopted soon afterwards, and remained popular throughout the 19th century, although some said that this radiation was entirely different from light.

The terms "chemical rays" and "heat rays" were eventually dropped in favor                    of ultraviolet and infrared radiation, respectively.  In 1878 the sterilizing                    effect of short-wavelength light by killing bacteria was discovered. By 1903 it              was known the most effective wavelengths were around 250 nm. In 1960, the effect of ultraviolet radiation on DNA was established.

The discovery of the ultraviolet radiation with wavelengths below 200 nm,               named "vacuum ultraviolet" because it is strongly absorbed by the oxygen in                air, was made in 1893 by the German physicist Victor Schumann.

APPLICATIONS:

The following information gives some uses of specific wavelength bands in the UV spectrum

13.5 nm: Extreme ultraviolet lithography

30–200 nm: Photo ionization, ultraviolet photoelectron spectroscopy, standard integrated circuit manufacture by photolithography

230–365 nm: UV-ID, label tracking, barcodes

230–400 nm: Optical sensors, various instrumentation

240–280 nm: Disinfection, decontamination of surfaces and water (DNA absorption has a peak at 260 nm)

200–400 nm: Forensic analysis, drug detection

270–360 nm: Protein analysis, DNA sequencing, drug discovery

280–400 nm: Medical imaging of cells

300–320 nm: Light therapy in medicine

300–365 nm: Curing of polymers and printer inks

350–370 nm: Bug zappers (flies are most attracted to light at 365 nm)

CHAPTER 5

ELECTROMAGNET

INTRODUCTION:

An electromagnet is a type of magnet in which the magnetic field is produced                by an electric current. Electromagnets usually consist of wire wound into a coil.               A current through the wire creates a magnetic field which is concentrated in                  the hole, denoting the center of the coil. The magnetic field disappears when                  the current is turned off. The wire turns are often wound around a magnetic        core made from a ferro- magnetic or ferri-magnetic material such as iron; the       magnetic core concentrates the magnetic flux and makes a more powerful magnet.

The main advantage of an electromagnet over a permanent magnet is that                       the magnetic field can be quickly changed by controlling the amount of                   electric current in the winding. However, unlike a permanent magnet that                   needs no power, an electromagnet requires a continuous supply of current to         maintain the magnetic field.

Electromagnets are widely used as components of other electrical devices,                   such as motors, generators, electromechanical solenoids, relays, loudspeakers,            hard disks, MRI machines, scientific instruments, and magnetic separation            equipment. Electromagnets are also employed in industry for picking up and           moving heavy iron objects such as scrap iron and steel.

HISTORY:

Danish scientist Hans Christian Orsted discovered in 1820 that electric currents         create magnetic fields. British scientist William Sturgeon invented the           electromagnet in 1824. His first electromagnet was a horseshoe-shaped piece                  of iron that was wrapped with about 18 turns of bare copper wire.                                   The iron was varnished to insulate it from the windings. When a current                        was passed through the coil, the iron became magnetized and attracted other              pieces of iron; when the current was stopped, it lost magnetization. Sturgeon            displayed its power by showing that although it only weighed around 200 grams,              it could lift around 4 kilograms when the current of a single-cell battery                         was applied. However, Sturgeon's magnets were weak because the uninsulated            wire he used could only be wrapped in a single spaced out layer around the core,     limiting the number of turns.

Beginning in 1830, US scientist Joseph Henry systematically improved and      popularized the electromagnet. By using wire insulated by silk thread, and             inspired by Schweigger's use of multiple turns of wire to make a galvanometer,              he was able to wind multiple layers of wire on cores, creating powerful                  magnets with thousands of turns of wire, including one that could support 936 kg.        The first major use for electromagnets was in telegraph sounders.

The magnetic domain theory of how ferromagnetic cores work was first proposed            in 1906 by French physicist Pierre-Ernest Weiss, and the detailed modern             quantum mechanical theory of ferromagnetism was worked out in the 1920s        by Werner Heisenberg, Lev Landau, Felix Bloch and others.

USES OF ELECTROMAGNETS:

Electromagnets are very widely used in electric and electromechanical devices, including:

Motors and generators

Transformers

Relays

Electric bells and buzzers

Loudspeakers and headphones

Actuators such as valves

Magnetic recording and data storage equipment

MRI machines

Scientific equipment such as mass spectrometers

Particle accelerators

Magnetic locks

Industrial lifting magnets

Magnetic levitation

Induction heating for cooking, manufacturing, and hyperthermia therapy

CHAPTER 6

RELAY

INTRODUCTION:

A relay is an electrically operated switch. Many relays use an electromagnet to mechanically operate a switch, but other operating principles are also used, such    as solid-state relays. Relays are used where it is necessary to control a circuit                  by a separate low-power signal, or where several circuits must be controlled                   by one signal. The first relays were used in long distance telegraph circuits as     amplifiers. Relays were used extensively in telephone exchanges and early        computers to perform logical operations.

A type of relay that can handle the high power required to directly control                      an electric motor or other loads is called a contactor. Solid-state relays                      control power circuits with no moving parts, instead using a semiconductor               device to perform switching. Relays with calibrated operating characteristics                and sometimes multiple operating coils are used to protect electrical                         circuits from overload or faults; in modern electric power systems these                functions are performed by digital instruments still called "protective relays".       Magnetic latching relays require one pulse of coil power to move their contacts               in one direction, and another, redirected pulse to move them back. Repeated              pulses from the same input have no effect. Magnetic latching relays are useful in applications where interrupted power should not affect the circuits that the relay               is controlling.

HISTORY:

In 1809 Samuel Thomas von Sömmerring designed an electrolytic relay as part               of his electrochemical telegraph. American scientist Joseph Henry is often               claimed to have invented a relay in 1835 in order to improve his version of       the electrical telegraph, developed earlier in 1831. It is claimed that English inventor Edward Davy "certainly invented the electric relay". in his electric telegraph c.1835. A simple device, which is now called a relay, was included                   in the original 1840 telegraph patent of Samuel Morse. The mechanism described       acted as a digital amplifier, repeating the telegraph signal, and thus allowing            signals to be propagated as far as desired. The word relay appears in the                   context of electromagnetic operations from 1860.

WORKING PRINCIPLE:

It works on the principle of an electromagnetic attraction. When the circuit of the        relay senses the fault current, it energizes the electromagnetic field which              produces the temporary magnetic field.

This magnetic field moves the relay armature for opening or closing the          connections.   The small power relay has only one contacts, and the high power          relay has two contacts for opening the switch.

The inner section of the relay is shown in the figure below. It has an iron core           which is wound by a control coil. The power supply is given to the coil                   through the contacts of the load and the control switch. The current flows                 through the coil produces the magnetic field around it.

Due to this magnetic field, the upper arm of the magnet attracts the lower                     arm. Hence close the circuit, which makes the current flow through the load.                    If the contact is already closed, then it moves oppositely and hence open the          contacts.

CONSTRUCTION:

The relay operates both electrically and mechanically. It consists electromagnetic         and sets of contacts which perform the operation of the switching. The             construction of relay is mainly classified into four groups. They are the                  contacts, bearings, electromechanical design, terminations and housing.

Contacts – The contacts are the most important part of the relay that                        affects the reliability. The good contact gives limited contact resistance and            reduced contact wear. The selection of the contact material depends upon                       the several factors like nature of the current to be interrupted, the magnitude                   of the current to be interrupted, frequency and voltage of operation.

Bearing – The bearing may be a single ball, multi-ball, pivot-ball and jewel           bearing. The single ball bearing is used for high sensitivity and low friction.                  The multi-ball bearing provides low friction and greater resistance to shock.

Electromechanical design – The electromechanical design includes the design              of the magnetic circuit and the mechanical attachment of core, yoke and                armature. The reluctance of the magnetic path is kept minimum for making                   the circuit more efficient. The electromagnet is made up of soft iron, and the coil     current is usually restricted to 5A and the coil voltage to 220V.

Terminations and Housing – The assembly of an armature with the magnet and           the base is made with the help of spring. The spring is insulated from the               armature by moulded blocks which provide dimensional stability. The fixed            contacts are usually spot welded on the terminal link.

CHAPTER 7

SMART PASS BOX CONTROLLER

INTRODUCTION:

Pass Box is used for material transfer from one  room to another  room. It consists           of a rectangular box with two doors. The two doors are interlocked such that                both doors cannot be open at the same time, to prevent pressure losses and cross contamination.

Pass Box is equipped with the UV lamp which is used for sterilization (Sterilization is      a term referring to any process that eliminates or kills all forms of microbial life, including transmissible agents present on a surface, contained in a fluid or in a compound such as biological culture media).

Pass box is provided with two interlocked doors i.e., when one door is open the          other door remains closed, in this way it prevents the direct contact between the           connected areas. A Reliable Interlock System prevents the two opposite doors                of the Pass Box from being opened at the same time, to maintain the stability                   of the clean room atmosphere

TYPES OF PASS BOX:

STATIC PASS BOX:

Static Pass Boxes are simple boxes mounted between two areas and also known              as “Passive Boxes”. The area having higher cleanliness level shall have the              higher pressure that helps to prevent the contamination in the sterile area.

Ultraviolet Light are also installed in the Pass Boxes to remove the contamination         that may enter during the transfer of the material. It should be replaced after                1000 thousand burning hours.

They are designed to work with minimal personnel movement while dynamic              pass box are used to pass through materials from an uncontrolled environment                 to a controlled environment. A static pass box should never be used to                     transfer the material between a clean room and a non clean room.

Pass boxes should be cleaned with disinfectants at regular intervals. The             efficiency of the pass boxes should be verified by validation. Validation                   should be done by exposing the known population of bacterial culture.

DYNAMIC PASSBOX:

Dynamic pass box works like an airlock or Laminar Air Flow Unit. Clean air             enters in the pass box and prevents the entrance of the contamination.                   Dynamic pass boxes may be castcade, sink or bubble type similar to the                     airlocks in pharmaceuticals.

A Dynamic Pass Box is a cubical box which has got the inter-locked doors                     on both sides. This protects the controlled environment from being polluted               during the transfer of material.

Dynamic Pass Boxes are used for the transfer of material from one area to                 another in isolation by means of mechanical/electromagnetic door interlocking        process that ensures opening of only one door at a time. This process helps in         avoiding cross contamination between two areas.

Pre-filters are attached to the Pass Box which are made from fiber glass media to   achieve high filtration efficiency and filtration accuracy of up to 5.0 microns.

These are used to block major particles from air within hatch being sucked by        blowers & then passing it through discharge filters thus enhancing life of             discharge filters.

UNIQUENESS OF THE INNOVATION:

The Automatic Door configuration includes one Automatic Door on the                   "clean" side and the other door on the "dirty" side. Automatic doors                               and ultrasonic sensors can be installed on both sides of the pass box on a                       custom basis.

The pass box consists of interlocking doors mechanism. The electromagnet                   gets activated through relay if no motion was detected by the ultrasonic sensor.                If the sensor detects any motion within a particular range it deactivates the            electromagnet and the door opens, then the impure material will be placed in                  the box. The UV light eliminates the impurities present and the material gets         collected from the other door.  If both ultrasonic sensors detects the motion simultaneously within the given range then the pass box remains in the                       initial state i.e., the two doors remain closed.

CONSTRUCTION:

Here, we use microcontroller which operates all the devices present in the                circuit. All devices are connected to the microcontroller. The UV light is placed        inside the box to remove the impurities if present. The electromagnets are fixed               at the two doors.  The electromagnet helps the door to remain in the closed             position during sterilization process. The ultrasonic sensors are installed at                     the two doors of the Pass Box. Whenever the ultrasonic sensor detects the                  motion it releases the door from the electromagnet using relay. The relay converts         the minimum input voltage to the maximum output voltage. Here we use the                    4-channel relay. The output of the ultrasonic sensor is given to the                           Arduino Mega Board (The sensor1 is connected to the Arduino digital                             pin 2(echo pin), 3(trig pin) and the sensor2 is connected to the Arduino                      Digital pin 5(echo pin), 4(trig pin)). We use only three relays in the given                         4-channel module. The relays are connected to the Arduino Mega board at                      the digital Pin 8(relay1), 9(relay2),10(relay3). The relays 1, 2 are connected                      to the two electromagnets and relay3 is connected to the UV light.

WORKING:

Whenever the ultrasonic sensor detects any motion in front of it then                               the sensor send its output as 1 to the microcontroller. Then                           microcontroller sends an input signal to the relay. The relay receives this                   signal. The relay converts the minimum input voltage to the maximum                       output voltage and gives supply to the two electromagnets and the UV light.              Based on the output given by the Microcontroller one of  the door will be                opened automatically and we can place the material inside the box                                 and close the door for sterilization process to take place, this process continues               for 1 minute. After the time lapse if the other ultrasonic sensor detects the                motion on it’s side the electromagnet gets demagnetized and the door gets opened automatically and we will collect the pure material.

BLOCK DIAGRAM:

CONCEPT AND OBJECTIVES:

In the process of material transfer from unclassified area to classified area                    pass boxes are used. Pass box is rectangular box with two doors. The two                   doors are interlocked such a way that two doors cannot be open at the same                 time, due to which pressure losses and bacterial contamination doesn’t take                 place throughout the material transferring process. The Automatic door                 operator eliminates touch contamination. Hands-free operation is especially well       suited for pharmaceutical and medical applications as it negates the need for                hand cleaning or re-gloving after using Pass box. Whenever the motion sensor       activated the pass box turns on else remains off to reduce the power               consumption. The prevention of simultaneous access of doors is been                      achieved by our Pass Box controller by ensuring that when one door for a                     Pass box is open then other door is closed.

PROJECT CODE:

#define trigPin1 3 // Door 2 Sensor (RIGHT)
#define echoPin1 2  // Door 2 Sensor (RIGHT)
#define trigPin2 4 // Door 1 Sensor (LEFT)
#define echoPin2 5 // Door 1 Sensor (LEFT)
int Door1=8;
int Door2=9;
int UV=10;

long duration, distance, RightSensor,LeftSensor;

void setup()
{
Serial.begin (9600);
pinMode(trigPin1, OUTPUT);
pinMode(echoPin1, INPUT);
pinMode(trigPin2, OUTPUT);
pinMode(echoPin2, INPUT);
pinMode(Door1, OUTPUT);
pinMode(Door2, OUTPUT);
pinMode(UV, OUTPUT);
}

void loop() 
{
SonarSensor(trigPin1, echoPin1);
RightSensor = distance;
SonarSensor(trigPin2, echoPin2);
LeftSensor = distance;

Serial.print(LeftSensor);
Serial.print(" - ");

Serial.println(RightSensor);

digitalWrite(Door1,HIGH);
digitalWrite(Door2,HIGH);
digitalWrite(UV,HIGH);

if(((LeftSensor<10)&&(RightSensor>10))||((LeftSensor>10)&&(RightSensor<10)))
{
if (LeftSensor<10)
{
  digitalWrite(Door1,LOW); //Door1 open
  digitalWrite(UV,LOW);   // UV Off
  delay(2000);
  digitalWrite(Door1,HIGH);
  digitalWrite(Door2,HIGH);
  digitalWrite(UV,HIGH);
  delay(10000);
}

if(RightSensor<10)
{
  
  digitalWrite(Door2,LOW);
  digitalWrite(UV,LOW);
  delay(2000);             //4time door will be opened for: 2 sec
  digitalWrite(Door1,HIGH);
  digitalWrite(Door2,HIGH);
  digitalWrite(UV,HIGH);
  delay(10000);             //time uv light on for: 10 sec
}
}
else
{
  digitalWrite(Door1,HIGH);
  digitalWrite(Door2,HIGH);
  digitalWrite(UV,HIGH);
}
}

void SonarSensor(int trigPin,int echoPin)
{
digitalWrite(trigPin, LOW);
delayMicroseconds(2);
digitalWrite(trigPin, HIGH);
delayMicroseconds(10);
digitalWrite(trigPin, LOW);
duration = pulseIn(echoPin, HIGH);
distance = (duration/2) / 29.1;
}

APPLICATIONS:

Space applications

Electronic industry

Chemical industry

Diagnostics labs

Pharmaceutical industry

Bio technology

Health care centers